SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0709

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques

Name: NETHRA HARIDASS

Reg Number: 1921260111

Assessment: Analytical Problem Solving

# Q1. Subnetting the 192.168.20.0/24 Network for Three Labs

Given: The university has the address block 192.168.20.0/24. Lab A requires a /26 subnet, Lab B a /27 subnet, and Lab C a /28 subnet. The subnets are assigned sequentially from the beginning of the /24 block without overlapping.

## Method and calculations

For IPv4, the number of host bits is 32 − prefix length. The total addresses are 2^(host bits), and the usable host addresses are total addresses − 2 because the network and broadcast addresses cannot be assigned to hosts.

| Lab | Prefix / Subnet mask | Block size | Network address | Valid host range | Broadcast address | Usable hosts |
| --- | --- | --- | --- | --- | --- | --- |
| Lab A | /26 — 255.255.255.192 | 64 | 192.168.20.0 | 192.168.20.1 – 192.168.20.62 | 192.168.20.63 | 2^6 − 2 = 62 |
| Lab B | /27 — 255.255.255.224 | 32 | 192.168.20.64 | 192.168.20.65 – 192.168.20.94 | 192.168.20.95 | 2^5 − 2 = 30 |
| Lab C | /28 — 255.255.255.240 | 16 | 192.168.20.96 | 192.168.20.97 – 192.168.20.110 | 192.168.20.111 | 2^4 − 2 = 14 |

Explanation: A /26 subnet contains 64 addresses, a /27 contains 32 addresses, and a /28 contains 16 addresses. The next subnet begins at the first address after the previous subnet’s broadcast address. Therefore, the three assignments do not overlap.

Final result: Lab A has 62 usable host addresses, Lab B has 30, and Lab C has 14. The address range 192.168.20.112–192.168.20.255 remains unassigned by these three lab allocations.

# Q2. Identifying the Subnet for Two Systems

Given: System 1 is 192.168.1.130 and System 2 is 192.168.1.190. Both use subnet mask 255.255.255.192, which is /26.

## Method and calculations

A /26 mask leaves 6 host bits. The block size in the last octet is 256 − 192 = 64. Therefore, the subnet ranges are 0–63, 64–127, 128–191, and 192–255.

| System | IP address | Subnet range | Network address | Valid host range | Broadcast address | Result |
| --- | --- | --- | --- | --- | --- | --- |
| System 1 | 192.168.1.130 | 192.168.1.128–192.168.1.191 | 192.168.1.128 | 192.168.1.129–192.168.1.190 | 192.168.1.191 | Valid host address |
| System 2 | 192.168.1.190 | 192.168.1.128–192.168.1.191 | 192.168.1.128 | 192.168.1.129–192.168.1.190 | 192.168.1.191 | Valid host address |

Usable host calculation: 32 − 26 = 6 host bits. Total addresses = 2^6 = 64; usable hosts = 64 − 2 = 62 per subnet.

Final result: Both IP addresses fall within the same subnet, 192.168.1.128/26. System 1 (192.168.1.130) and System 2 (192.168.1.190) are both valid host addresses. Each subnet provides 62 usable host addresses.

# Q3. Department Subnet Allocation in 172.16.0.0/16

Given: The organization has 172.16.0.0/16. The required subnet sizes are HR /25, Sales /26, IT /27, and Admin /28. The subnets are allocated sequentially from the start of the address block.

## Method and calculations

The host count is calculated using 2^(32 − prefix length) total addresses and subtracting 2 for the IPv4 network and broadcast addresses. Each next subnet starts at the next available boundary after the preceding subnet.

| Department | Prefix / Subnet mask | Network address | Valid host range | Broadcast address | Usable hosts |
| --- | --- | --- | --- | --- | --- |
| HR | /25 — 255.255.255.128 | 172.16.0.0 | 172.16.0.1 – 172.16.0.126 | 172.16.0.127 | 2^7 − 2 = 126 |
| Sales | /26 — 255.255.255.192 | 172.16.0.128 | 172.16.0.129 – 172.16.0.190 | 172.16.0.191 | 2^6 − 2 = 62 |
| IT | /27 — 255.255.255.224 | 172.16.0.192 | 172.16.0.193 – 172.16.0.222 | 172.16.0.223 | 2^5 − 2 = 30 |
| Admin | /28 — 255.255.255.240 | 172.16.0.224 | 172.16.0.225 – 172.16.0.238 | 172.16.0.239 | 2^4 − 2 = 14 |

Total usable addresses allocated = 126 + 62 + 30 + 14 = 232.

Remaining unused range: 172.16.0.240 through 172.16.255.255, inclusive, assuming only the four listed subnets are allocated. This range includes the unused addresses at the end of the 172.16.0.0/24 portion and all subsequent addresses within the original /16 block.

Final result: HR receives 172.16.0.0/25, Sales receives 172.16.0.128/26, IT receives 172.16.0.192/27, and Admin receives 172.16.0.224/28. These allocations are contiguous and non-overlapping.

# Q4. Subnetting 172.16.10.0/24 Using /27

Given: The original network is 172.16.10.0/24 and the subnet mask is /27 (255.255.255.224). The address to identify is 172.16.10.78; we must also check 172.16.10.95.

## Method and calculations

Bits borrowed for subnetting = 27 − 24 = 3 bits.

Number of subnets = 2^3 = 8.

Host bits remaining = 32 − 27 = 5 bits.

Total addresses per subnet = 2^5 = 32; usable host addresses = 32 − 2 = 30.

Block size in the last octet = 256 − 224 = 32.

The eight /27 subnet network addresses within 172.16.10.0/24 are .0, .32, .64, .96, .128, .160, .192, and .224.

| Item | Result |
| --- | --- |
| Subnet containing 172.16.10.78 | 172.16.10.64/27 (range .64–.95) |
| Network address | 172.16.10.64 |
| Valid host range | 172.16.10.65 – 172.16.10.94 |
| Broadcast address | 172.16.10.95 |
| Does 172.16.10.95 fall in the same subnet range? | Yes, it is within the .64–.95 subnet block, but it is the broadcast address and cannot be assigned to a host. |

Final result: The /24 network is divided into 8 subnets, each with 30 usable host addresses. The IP 172.16.10.78 belongs to 172.16.10.64/27. The address 172.16.10.95 is in that same subnet block but is its broadcast address, not a valid host address.

# Q5. IPv6 Address Compression, Expansion, and /64 Prefix

Given IPv6 address: 2001:0db8:0000:0000:0000:ff00:0042:8329. The prefix length is /64.

## Method and calculations

IPv6 addresses contain eight groups (hextets) of four hexadecimal digits, separated by colons. Leading zeros in a hextet may be removed. One continuous sequence of zero hextets may be replaced by :: once in an address.

1. Compressed form: 2001:db8::ff00:42:8329

2. Expanded full form: 2001:0db8:0000:0000:0000:ff00:0042:8329

3. Network prefix with /64: 2001:0db8:0000:0000::/64 (equivalently, 2001:db8:0:0::/64). The first 64 bits—the first four hextets—identify the network prefix.

4. Host/interface identifier bits = 128 − 64 = 64 bits.

5. Number of possible addresses in the /64 prefix = 2^64 = 18,446,744,073,709,551,616 addresses.

IPv6 interpretation: Unlike traditional IPv4 subnet calculations, IPv6 does not use a broadcast address, and the address count should not be reduced by two. The 2^64 figure is the total number of addresses in this /64 prefix.

Final result: The shortest compressed address is 2001:db8::ff00:42:8329, its expanded form matches the original address, the /64 network prefix is 2001:db8:0:0::/64, and it contains 2^64 possible IPv6 addresses.

# Summary of Final Answers

| Question | Key result |
| --- | --- |
| Q1 | Lab A: 192.168.20.0/26 (62 hosts); Lab B: 192.168.20.64/27 (30 hosts); Lab C: 192.168.20.96/28 (14 hosts). |
| Q2 | Both systems are in 192.168.1.128/26; valid host range .129–.190; broadcast .191; 62 usable hosts. |
| Q3 | HR: 172.16.0.0/25; Sales: 172.16.0.128/26; IT: 172.16.0.192/27; Admin: 172.16.0.224/28; unused range begins at 172.16.0.240. |
| Q4 | 8 subnets, 30 usable hosts each; .78 belongs to 172.16.10.64/27; .95 is that subnet's broadcast address. |
| Q5 | Compressed: 2001:db8::ff00:42:8329; prefix: 2001:db8:0:0::/64; total addresses: 2^64. |

Note: The subnet assignments in Questions 1 and 3 use sequential, non-overlapping allocation because no specific starting subnet was prescribed for each lab or department.